基本原理

基于普通摄像头的视觉抓取，使用了视觉处理方法，通过识别定位框的位置和物块的相对位置，获得物块到机械臂的位置。

颜色识别原理

颜色的参数分别是：色调（H），饱和度（S），明度（V）。

色调H

用角度度量，取值范围为0°～360°，从红色开始按逆时针方向计算，

饱和度S

饱和度S表示颜色接近光谱色的程度。一种颜色，可以看成是某种光谱色与白色混合的结果。

其中光谱色所占的比例愈大，颜色接近光谱色的程度就愈高，颜色的饱和度也就愈高。

饱和度高，颜色则深而艳。

光谱色的白光成分为0，饱和度达到最高。

通常取值范围为0%～100%，值越大，颜色越饱和。

明度V

明度表示颜色明亮的程度，

对于光源色，明度值与发光体的光亮度有关；

对于物体色，此值和物体的透射比或反射比有关。

通常取值范围为0%（黑）到100%（白）。

RGB和CMY颜色模型都是面向硬件的，而HSV（Hue Saturation Value）颜色模型是面向用户的。

文件说明

script

主程序main调用 arm.py和camera.py，相机模块调用颜色识别和定位框识别模块

main.py---->arm.py

----->camera.py---->color_detection.py---> cvwin.py

----->plate_detection.py---> cvwin.py

cvwin.py  单独的cv2相关的窗口显示线程

launch

aiarm-client-controller.launch     虚拟机上使用的

aiarm-controller.launch          网关上使用的，一般使用这个

参数说明

#定位板相关参数 loc_plate=[141,192,465,386] #初始值，机械臂自己根据plate_detection.py文件中自己查找并更新此参数 单位像素 不用设置 loc_plate_act=[0.153,0.173,0.092] #定位板实际长度 单位 m 定位板框实际长度 上底 下底 高[0.155,0.173,0.093] loc_plate_act_origin=[0.23,-0.0148] #定位板原点偏移 单位 m 定位板相对于base_link的x,y方向偏移量 loc_x_off_mx=25 loc_x_off_mi=9 loc_y_off_mx=0 loc_opt_par_x=1.0 #X方向的优化参数 loc_opt_par_y=1.1 #Y方向的优化参数 ''' loc_plate 单位 pix 640*480像素 定位板框 左上角x,y 和 右下角x,y loc_plate_act 单位 m 定位板框实际长度 上底 下底 高[0.147,0.182,0.082] loc_x_off_mx 单位 pix 相机倾斜视角X像素偏移 最大值 loc_y_off_mx 单位 pix 相机倾斜视角y像素偏移 最大值 loc_plate_origin 单位 pix cv2下像素原点 loc_plate_act_origin 单位 m 实际定位原点 相对base_link ''' #机械臂 arm_g_height=0.1 #拍照位 # arm_cam_pose=[0.1572271238, 0.01332804092, 0.2300313884, 0.04640923861, 0.4895623458, 0.00796998209, 0.87069591217] arm_cam_joint=[0.11535425216048287, -0.616704188215822, 1.40664924293023, 1.8002512210459807, 0.17097973936214012] #过渡位 arm_trans_joint=[5.46315397666901e-05, 0.15632741105576997, 1.284452648106851, 0.9141770383249787, -9.986523001948356e-05]

坐标系说明

操作步骤

机械臂走到相机拍照位

1、启动launch启动文件

启动一个ssh终端

$ roslaunch marm_visual_control aiarm-controller.launch ... logging to /home/zonesion/.ros/log/24f02082-b08b-11ec-885a-b1e0c15c78ec/roslaunch-rk3399-9153.log Checking log directory for disk usage. This may take awhile. Press Ctrl-C to interrupt Done checking log file disk usage. Usage is <1GB. started roslaunch server http://rk3399:46050/ SUMMARY ======== PARAMETERS * /grasp_filter_test/arm: manipulator [ INFO] [1648688509.308670709]: MoveGroup context using planning plugin ompl_interface/OMPLPlanner [ INFO] [1648688509.309991661]: MoveGroup context initialization complete You can start planning now!

2、运行到拍照位

启动一个ssh终端

zonesion@rk3399:~$ cd catkin_ws/src/marm_visual_control/script/ zonesion@rk3399:~/catkin_ws/src/marm_visual_control/script$ python main.py Failed to import pyassimp, see https://github.com/ros-planning/moveit/issues/86 for more info [ INFO] [1648688601.580013535]: Loading robot model 'marm'... [ INFO] [1648688601.580419243]: No root/virtual joint specified in SRDF. Assuming fixed joint [ INFO] [1648688604.750564185]: Ready to take commands for planning group manipulator. ('red color_par', (156, 51, 190, 190, 255, 255)) ('green color_par', (50, 102, 158, 71, 200, 200)) libpng warning: Incorrect sBIT chunk length set cam number 0 [INFO] [1648688606.222780]: Open the fixture ^CEXIT APP zonesion@rk3399:~/catkin_ws/src/marm_visual_control/script$

运行main.py脚本，当机械臂动作后，退出程序

颜色识别标定

使用color_detection.py文件

1、机械臂走到拍照位

2、标定颜色

zonesion@rk3399:~/catkin_ws/src/marm_visual_control/script$ python color_detection.py -c yellow

如标定黄色，后面跟随参数为yellow即可。如下这样标定所有颜色，

右边这样也不影响，主要黄色的木块能够识别出来即可。后面的camera应用中会将不在定位框内的进行清楚

将颜色识别参数写入到color_param.

第一组颜色识别参数用于定位物块的位置

第二组颜色识别参数用于定位定位板框位置，为了提高定位框识别精度用的。

同时保证两个识别框数据稳定，不然会早层定位数据失败

定位框识别标定

同样机械臂走到拍照位。

1、运行plate_detection.py

zonesion@rk3399:~/catkin_ws/src/marm_visual_control/script$ python plate_detection.py libpng warning: Incorrect sBIT chunk length

2、调整参数

识别出四个角，同时物体显示为白色，说参数就可以

通常阈值2不用调，只需要调整第一个值

如上参数为：120,300

3、调整色块色域参数

zonesion@rk3399:~/catkin_ws/src/marm_visual_control/script$ python color_detection.py -c yellow -t 1

('yellow color_par', [25, 45, 45, 30, 255, 255])

('yellow', [[598, 255, 0.0], [277, 149, 24.4439544

主要黄色的木块能够整个识别即可。这样

这样就可以进行相应的取反颜色。

主程序说明

aiarmcontrol():

def aiarmcontrol(): # win=[98,420,103,560] #选择剪裁窗口 win = [] rospy.init_node("AIARM_NODE", log_level=rospy.INFO) aiarm=AiArm(-40, ("red","green","blue"),win,loc_plate,loc_plate_act,loc_plate_act_origin,loc_x_off_mx,loc_x_off_mi, loc_y_off_mx,this.color_param,this.bin_param) #初始化AIarm aiarm.all_gohome() aiarm.set_joint_value_target(arm_cam_joint) time.sleep(1) aiarm.plate_to_base() for i in aiarm.rec_cla_dict.keys(): point=aiarm.rec_cla_dict[i]["pos"] point.insert(0,i) aiarm.block_pos.append(point) while len(aiarm.block_pos)>0 or len(aiarm.solutions)>0: while len(aiarm.solutions)>0: solution=aiarm.solutions[0] # print(solution) __msg="Grabbing "+solution[0]+" wood" rospy.loginfo(__msg) joint_positions = solution[1] aiarm.setGripper(False) time.sleep(1) aiarm.set_joint_value_target(joint_positions) time.sleep(0.5) if solution[3]>45: solution[3]=90-solution[3] aiarm.rotate_gripper(solution[3],arm_cam_joint[4]) else: aiarm.rotate_gripper(-solution[3],arm_cam_joint[4]) joint_positions = solution[2] aiarm.set_arm_joint_value_target(joint_positions) time.sleep(0.5) aiarm.setGripper(True) time.sleep(1) joint=aiarm.get_joints() arm_trans_joint[0]=joint[0] #不旋转1号舵机抬起 aiarm.set_joint_value_target(arm_trans_joint) time.sleep(0.5) aiarm.arm_goHome() del aiarm.solutions[0] time.sleep(0.5) aiarm.all_gohome() time.sleep(1) print('EXIT APP') sys.exit()

aiarm=AiArm(-40,("red","green"),win,loc_plate,loc_plate_act,loc_plate_act_origin,loc_x_off_mx,loc_x_off_mi,

loc_y_off_mx,this.color_param,this.bin_param)           #初始化AIarm如上，这里定义可aiarm机械臂的

1、夹具打开角度为-40时，关闭也就是-40-80=-120

2、定义此机械臂进行"red","green"，红色和绿色两个色块的抓取。如果只抓取一个色块，定义成("red",)这样。这里的逗号不要省掉。

3、win参数，如果需要截取一部分视频窗口，可配置此参数，# win=[98,420,103,560]      #选择剪裁窗口

4、其他为主要参数，见参数解释

启动文件

关闭所有终端，使用启动程序启动以上程序

注意事项

1、如果蓝色和绿色四个角识别不是很好，主要是光线不是很好，加上木块材质反光，导致四个角的颜色识别参数不一样，演示时候可以使用红色放在四角，红色识别比较好，蓝色和绿色就固定放在一侧即可。